VNet-to-VNet VPN using Azure VPN Gateway

🔷 Assessment Objective

Connect two virtual networks using site-to-site style VPN inside Azure (VNet-to-VNet).

Concepts –

🔷 Core Idea

You are creating a secure, encrypted tunnel between two isolated networks inside Azure using Azure VPN Gateway.

👉 Think of it as:

Two private office networks connected over the internet using IPSec VPN

GatewaySubnet (Critical Component)

10.0.255.0/27

10.1.255.0/27

🔑 Concept

This is a dedicated subnet reserved for VPN gateway instances

❗ Why special?

Azure deploys managed gateway VMs internally

You cannot deploy your own VMs here

Must be named exactly: GatewaySubnet

👉 Think:

“This is where Azure places the VPN device”

Public IP (Behind the Scenes)

Each VPN Gateway has:

A Public IP

🔑 Concept

Even though VNets are private:

Gateways communicate over the public internet using encrypted tunnels

The VPN Tunnel (Connection)

VPN Gateway 1  <------VPN------>  VPN Gateway 2

🔑 Concept

This is a site-to-site IPSec tunnel

🔄 Packet Flow

Example: VM1 → VM2

VM1 sends packet → 10.1.1.4

Azure routing detects:

Destination is remote VNet

Packet sent to:
→ VPN Gateway 1

Gateway:

Encrypts packet

Sends via internet

VPN Gateway 2:

Decrypts packet

Delivered to VM2

Why we have gateway subnet if we have vpn gateway?

VPN Gateway is a managed cluster of gateway VMs (controlled by Azure) that performs encryption, routing, and tunneling.

These VMs need IP addresses, They must be placed inside your Vnet GatewaySubnet.

🧱 Architecture Overview

VNet-1 (10.0.0.0/16)              VNet-2 (10.1.0.0/16)
   |                                   |
Subnet1 (VM1)                      Subnet1 (VM2)
   |                                   |
GatewaySubnet                      GatewaySubnet
   |                                   |
VPN Gateway 1  <------VPN------>  VPN Gateway 2

🚀 Step-by-Step Lab (Azure Portal)

✅ Step 1: Create Resource Group

Name: rg-vnet-vpn-lab

Region: Choose same region (e.g., Central India)

✅ Step 2: Create VNet-1

Screenshots:

Figure: vnet1.jpeg

Name: vnet1

Address space: 10.0.0.0/16

Subnets:

subnet1 → 10.0.1.0/24

GatewaySubnet → 10.0.255.0/27 (must be named exactly this)

✅ Step 3: Create VNet-2

Screenshots:

Figure: connection of vnet2.jpeg

Name: vnet2

Address space: 10.1.0.0/16

Subnets:

subnet1 → 10.1.1.0/24

GatewaySubnet → 10.1.255.0/27

✅ Step 4: Create VPN Gateway for VNet-1

Screenshots:

Figure: resource for vpn.jpeg

Go to:
👉 Create Resource → Virtual Network Gateway

Name: vnet1-gateway

Gateway type: VPN

VPN type: Route-based

SKU: VpnGw1 (basic lab)

VNet: vnet1

Public IP: Create new (pip-vnet1-gw)

⏳ Takes ~20–30 minutes

✅ Step 5: Create VPN Gateway for VNet-2

Name: vnet2-gateway

Same config, attach to vnet2

Public IP: pip-vnet2-gw

⏳ Again ~20–30 minutes

✅ Step 6: Create Connection (VNet1 → VNet2)

Screenshots:

Figure: connection vnetGateway.jpeg

Go to:
👉 vnet1-gateway → Connections → + Add

Name: vnet1-to-vnet2

Connection type: VNet-to-VNet

Second virtual network gateway: vnet2-gateway

Shared key (PSK): Azure123

✅ Step 7: Create Reverse Connection (VNet2 → VNet1)

Screenshots:

Figure: connection device of vnet1.jpeg

Name: vnet2-to-vnet1

Same shared key: Azure123

✅ Step 8: Deploy Test VMs

VM1 in VNet1

Name: vm1

Subnet: subnet1

OS: Ubuntu / Windows

VM2 in VNet2

Name: vm2

Subnet: subnet1

✅ Step 9: Test Connectivity

Screenshots:

Figure: step 9 ping command for vm1.jpeg

Figure: ping from vm2 to vm1 on private network.jpeg

From VM1:

ping 10.1.1.x   # VM2 private IP

From VM2:

ping 10.0.1.x

✔ If ping works → VPN is successful

🔍

🔸 Why GatewaySubnet?

Dedicated subnet for VPN gateway

Cannot deploy anything else there

🔸 Why VPN Gateway?

Provides encrypted tunnel (IPSec/IKE)

Needed for cross-network secure communication

Overall Architecture Screenshot:

Figure: overall.jpeg